How to add an (S)VGA driver to XFree86

Copyright (c) 1993, 1994 David E. Wexelblat

<dwex@XFree86.org>

Issue 1.3 - May 29, 1994

1.  Introduction

Adding support for a new SVGA chipset to XFree86 is a challenging project for

someone who wants to learn more about hardware-level programming.  It can be

fraught with hazards (in particular, crashing the machine is all too common).

But in the end, when the server comes up and functions, it is immensely sat-

isfying.

Adding support for an SVGA chipset does not change any of the basic function-

ing of the server.  It is still a dumb 8-bit PseudoColor server or 1-bit

StaticGray server.  Adding support for new hardware (e.g. accelerated chips)

is a major undertaking, and is not anywhere near formalized enough yet that

it can be documented.

Nonetheless, the driver-level programming here is a good introduction.  And

can well be the first step for adding support for an accelerated chipset, as

many are SVGA-supersets.  Writing an SVGA-level driver for the chipset can

provide a stable development platform for making use of new features (in

fact, this has been done for the S3, Cirrus, and WD accelerated chipsets, for

internal use as the accelerated servers are developed for XFree86 2.0).

Now let's get down to it.  In addition to this documentation, a stub driver

has been provided.  This should provide you a complete framework for your new

driver.  Don't let the size of this document persuade you that this is an

overly difficult task.  A lot of work has been put into making this document

as close to complete as possible; hence it should, in theory, be possible to

use this as a cookbook, and come out with a working driver when you reach the

end.  I do advise that you read it all the way through before starting.

2.  Getting Started

The first step in developing a new driver is to get the documentation for

your chipset.  I've included a list of vendor contact information that I have

collected so far (it's far from complete, so if you have any that isn't on

the list, please send it to me).  You need to obtain the databook for the

chipset.  Make sure that the person you speak to is aware that you intend to

do register-level programming (so they don't send you the EE-style

datasheet).  Ask for any example code, or developer's kits, etc.  I've

learned that at the SVGA level, in general, a databook that lists and

describes the registers is the most you can hope to find.

If you are not familiar with VGA register-level programming, you should get

(and read!) a copy of Richard Ferraro's bible (see references below).  The

best way to understand what is happening in the server is to study the work-

ings of the monochrome server's ``generic'' server, and compare it with the

documentation in Ferraro's book (be aware that there are a few errors in the

book).  You can find the generic-VGA-register handling functions in the file

``vgaHW.c''.

Once you understand what's happening in the generic server, you should study

one or more of the existing SVGA drivers.  Obtain the databook for a sup-

ported SVGA chipset, and study the documentation along with the code.  When

you have a good understanding of what that driver does over and above the

generic VGA, you will know what information you need to obtain from the data-

book for the new chipset.  Once you have this information, you are ready to

begin work on your new driver.

3.  Directory Tree Structure

Here is an outline of the directory tree structure for the source tree.  Only

directories/files that are relevant to writing a driver are presented.  The

structure for the Link Kit is presented below.

xc/config/cf/

site.def

Local configuration customization

xf86site.def

XFree86 local configuration customization

xc/programs/Xserver/hw/xfree86/

The server source

common/

Files common to all of the server (XF86Config parser,

I/O device handlers, etc)

xf86.h

Contains the `ScrnInfoRec' data structure

xf86_Option.h

Contains option flags

compiler.h

Contains in-line assembler macros and

utility functions

os-support/

OS-support layer

assyntax.h

Contains macro-ized assembler mnemonics

xf86_OSlib.h

OS-support includes, defines, and proto-

types

LinkKit/

site.def.LK

Template for Link Kit site.def

vga256/

256-color VGA server directories

vga/

The generic VGA handling code

vga.h

Contains the

`vgaVideoChipRec' and `vgaH-

WRec' data structures

vgaHW.c

Contains the generic-VGA-reg-

ister handling functions

vgaHWInit(), vgaHWSave() and

vgaHWRestore().

drivers/

Contains the SVGA driver subdirectories.

Each contains an Imakefile, a .c file for

the driver, and a .s file for the bank-

switching functions.

vga2/

The monochrome vga server directories.  Most of the

files are linked from vga256, and the differences

handled by conditional compilation.

drivers/

The SVGA driver subdirectories.  The

`generic' VGA driver is also located

here.

vga16/

The 16-color vga server directories.  Most of the

files are linked from vga256, and the differences

handled by conditional compilation.

drivers/

The SVGA driver subdirectories.

VGADriverDoc/

This documentation and the stub driver.

The Link Kit is usually installed in /usr/X11R6/lib/Server.  The Link Kit

contains everything that is needed to relink the server.  It is possible to

write a new driver and build a new server without having even the server

source installed.

Server/

site.def

Local configuration customization

include/

All of the include files listed under the `common'

directory above

drivers/

All of the SVGA drivers

vga2/

The SVGA driver subdirectories.

vga16/

The SVGA driver subdirectories.

vga256/

The SVGA driver subdirectories.

VGADriverDoc/

The directory with this documentation and the stub

driver.  `vgaHW.c' is also copied here, for reference

(it is not built as part of the Link Kit).

4.  Setting Up The Build Information

This section describes the peripheral configuration and build steps that must

be performed to set up for your new driver.  The steps are the same whether

you are building from the source tree of from the Link Kit; only the loca-

tions of the files is different.  Here are the configuration steps that must

be followed:

1.        Choose the name for your driver subdirectory and data structures.

Since the current driver scheme allows (in fact, encourages) putting

drivers for multiple related chipsets in a single driver, it is usually

best to use the vendor name, rather than a chipset version.  The fact

that older XFree86 drivers do not follow this convention should not

deter you from using it now - most of that code was developed before

the driver interface had been made flexible and extensible.

For this documentation, we'll use chips from the SuperDuper Chips ven-

dor.  Hence, we'll use `sdc' for the name of the driver.

2.        Decide whether your driver will support the color server, the

monochrome server, or both.  For this documentation, we will assume

that both the color and monochrome servers will be supported.  If you

intend to support only the color server, the steps for the monochrome

server can be ignored.  If you intend to support only the monochrome

server, the steps for the color server listed should be performed for

the monochrome server, and the monochrome steps ignored.  Most of the

existing drivers support only the color or both servers; the

``generic'' driver is the only driver (currently) that supports just

the monochrome server.

3.        Create your driver directories:

o If you are working in the source tree, create the following direc-

tories:

xc/programs/Xserver/hw/xfree86/vga256/drivers/sdc

xc/programs/Xserver/hw/xfree86/vga16/drivers/sdc

xc/programs/Xserver/hw/xfree86/vga2/drivers/sdc

o If you are working in the Link Kit, create the following directo-

ries:

/usr/X11R6/lib/Server/drivers/vga256/sdc

/usr/X11R6/lib/Server/drivers/vga16/sdc

/usr/X11R6/lib/Server/drivers/vga2/sdc

4.        Set up the Imakefile parameters to cause your driver to be built:

o If you are working in the source tree:

1.  Edit the file xc/config/cf/xfree86.cf, and add `sdc' to the

list for the definitions for `XF86Vga256Drivers',

`XF86Vga16Drivers' and `XF86Vga2Drivers'.  You should put

`sdc' just before `generic' in the list (i.e. second last),

to ensure that none of the other driver's probe functions

incorrectly detect the `sdc' chipset .

2.  Edit the file xc/config/cf/xf86site.def, and add the same

entries in this file (this is just a comment that shows the

default values).

3.  Edit the site.def.LK file in xc/pro-

grams/Xserver/hw/xfree86/LinkKit/, and add the same entries

in this file.  This is the prototype `site.def' file that

will be installed in the Link Kit.

o If you are working in the Link Kit, edit the file

/usr/X11R6/lib/Server/site.def, and add `sdc' to the

`XF86Vga256Drivers', `XF86Vga16Drivers' and `XF86Vga2Drivers' def-

initions as described in (a) above.

5.        Now copy the prototype files into your new directories:

o If you are working in the source tree, copy the `stub' files as

follows (directories are below xc/programs/Xserver):

Imakefile.stub =>

hw/xfree86/vga256/drivers/sdc/Imakefile

stub_driver.c =>

hw/xfree86/vga256/drivers/sdc/sdc_driver.c

stub_bank.s =>

hw/xfree86/vga256/drivers/sdc/sdc_bank.s

Imakefile.stub =>

hw/xfree86/vga16/drivers/sdc/Imakefile (then edit this

Imakefile and make the changes described in the com-

ments).

Imakefile.stub =>

hw/xfree86/vga2/drivers/sdc/Imakefile (then edit this

Imakefile and make the changes described in the com-

ments).

o If you are working in the Link Kit, copy the `stub' files as fol-

lows:

Imakefile.stub =>

/usr/X11R6/lib/Server/drivers/vga256/sdc/Imakefile

stub_driver.c =>

/usr/X11R6/lib/Server/drivers/vga256/sdc/sdc_driver.c

stub_bank.s =>

/usr/X11R6/lib/Server/drivers/vga256/sdc/sdc_bank.s

Imakefile.stub =>

/usr/X11R6/lib/Server/drivers/vga16/sdc/Imakefile

(then edit this Imakefile and make the changes

described in the comments).

Imakefile.stub =>

/usr/X11R6/lib/Server/drivers/vga2/sdc/Imakefile (then

edit this Imakefile and make the changes described in

the comments).

6.        Edit each of the files you've just copied, and replace `stub'

with `sdc' and `STUB' with `SDC' wherever they appear.

That's all the prep work needed.  Now it's time to work on the actual driver.

5.  The Bank-Switching Functions

The normal VGA memory map is 64k starting at address 0xA0000.  To access more

than 64k of memory, SuperVGA chipsets implement ``bank switching'' - the

high-order address bits are used to select the bank of memory in which opera-

tions will take place.  The size and number of these banks varies, and will

be spelled out in the chipset documentation.  A chipset will have zero, one

or two bank registers.  Likely the ONLY case of zero bank registers is a

generic VGA, and hence is not a concern.

Note that some of the newer chipsets (e.g. Trident 8900CL, Cirrus) allow for

a linear mapping of the video memory.  While using such a scheme would

improve the performance of the server, it is not currently supported.  Hence

there is no way to use such features for a new chipset.

Most SVGA chipsets have two bank registers.  This is the most desirable

structure (if any banking structure can be called ``desirable''), because

data can be moved from one area of the screen to another with a simple `mov'

instruction.  There are two forms of dual-banking - one where the two bank

operations define a read-only bank and a write-only bank, and one with two

read/write windows.  With the first form, the entire SVGA memory window is

used for both read a write operations, and the two bank registers determine

which bank is actually used (e.g. ET3000, ET4000).  With the second form, the

SVGA memory window is split into two read/write banks, with each bank pointer

being used to control one window.  In this case, one window is used for read

operations and the other for write operations (e.g. PVGA1/Western Digital,

Cirrus).

A chipset that has a single bank register uses that one bank for both read

and write access.  This is problematic, because copying information from one

part of the screen to another requires that the data be read in, stored, and

then written out.  Fortunately, the server is able to handle both one-bank

and two-bank chipsets; the determination of behavior is defined by an entry

in the driver data structure described below.

A driver requires that three assembly-language functions be written, in the

file `sdc_bank.s'.  These functions set the read bank - SDCSetRead(), the

write bank - SDCSetWrite(), and set both banks - SDCSetReadWrite().  For a

chipset with only one bank, all three will be declared as entry points to the

same function (see the ``tvga8900'' driver for an example).

The functions are fairly simple - the bank number is passed to the function

in register %al.  The function will shift, bitmask, etc - whatever is

required to put the bank number into the correct form - and then write it to

the correct I/O port.  For chipsets where the two banks are read-only HERE

and write-only, the SetReadWrite() function will have to do this twice - once

for each bank.  For chipsets with two independent read/write windows, the

SetReadWrite() function should use the same bank as the SetWrite() function.

A special note - these functions MUST be written in the macroized assembler

format defined in the header file ``assyntax.h''.  This will ensure that the

correct assembler code will be generated, regardless of OS.  This macroized

format currently supports USL, GNU, and Intel assembler formats.

That's all there is to the banking functions.  Usually the chipset reference

will give examples of this code; if not, it is not difficult to figure out,

especially using the other drivers as examples.

6.  The Driver Itself

Now it's time to get down to the real work - writing the major driver func-

tions in the files sdc_driver.c.  First, an overview of what the responsibil-

ities of the driver are:

1.          Provide a chipset-descriptor data structure to the server.

This data structure contains pointers to the driver functions and some

data-structure initialization as well.

2.          Provide a driver-local data structure to hold the contents of

the chipset registers.  This data structure will contain a generic part

and a driver-specific part.  It is used to save the initial chipset

state, and is initialized by the driver to put the chipset into differ-

ent modes.

3.          Provide an identification function that the server will call to

list the chipsets that the driver is capable of supporting.

4.          Provide a probe function that will identify this chipset as

different from all others, and return a positive response if the

chipset this driver supports is installed, and a negative response oth-

erwise.

5.          Provide a function to select dot-clocks available on the board.

6.          Provide functions to save, restore, and initialize the driver-

local data structure.

7.          Provide a function to set the starting address for display in

the video memory.  This implements the virtual-screen for the server.

8.          Perhaps provide a function for use during VT-switching.

9.          Perhaps provide a function to check if each mode is suitable

for the chipset being used.

Before stepping through the driver file in detail, here are some important

issues:

1.          If your driver supports both the color and monochrome servers,

you should take care of both cases in the same file.  Most things are

the same - you can differentiate between the two with the MONOVGA

#define.  If the 16 color server is supported, code specific to it can

be enabled with the XF86VGA16 #define.  In most cases  it is sufficient

to put the following near the top of the stub_driver.c file:

#ifdef XF86VGA16

#define MONOVGA

#endif

2.          The color server uses the SVGA's 8-bit packed-pixel mode.  The

monochrome and vga16 servers uses the VGA's 16-color mode (4 bit-

planes).  Only one plane is enabled for the monochrome server.

3.          It is possible for you to define your monochrome driver so that

no bank-switching is done.  This is not particularly desirable, as it

yields only 64k of viewing area.

Keeping these things in mind, you need to find the registers from your SVGA

chipset that control the desired features.  In particular, registers that

control:

1.          Clock select bits.  The two low-order bits are part of the

standard Miscellaneous Output Register; most SVGA chipsets will include

1 or 2 more bits, allowing the use of 8 or 16 discrete clocks.

2.          Bank selection.  The SVGA chipset will have one or two regis-

ters that control read/write bank selection.

3.          CRTC extensions.  The standard VGA registers don't have enough

bits to address large displays.  So the SVGA chipsets have extension

bits.

4.          Interlaced mode.  Standard VGA does not support interlaced dis-

plays.  So the SVGA chipset will have a bit somewhere to control inter-

laced mode.  Some chipsets require additional registers to be set up to

control interlaced mode

5.          Starting address.  The standard VGA only has 16 bits in which

to specify the starting address for the display.  This restricts the

screen size usable by the virtual screen feature.  The SVGA chipset

will usually provide one or more extension bits.

6.          Lock registers.  Many SVGA chipset prevent modification of

extended registers unless the registers are first ``unlocked''.  You

will need to disable protection of any registers you will need for

other purposes.

7.          Any other facilities.  Some chipset may, for example, require

that certain bits be set before you can access extended VGA memory

(beyond the IBM-standard 256k).  Or other facilities; read through all

of the extended register descriptions and see if anything important

leaps out at you.

If you are fortunate, the chipset vendor will include in the databook some

tables of register settings for various BIOS modes.  You can learn a lot

about what manipulations you must do by looking at the various BIOS modes.

6.1  Multiple Chipsets And Options

It is possible, and in fact desirable, to have a single driver support multi-

ple chipsets from the same vendor.  If there are multiple supported chipsets,

then you would have a series of #define's for them, and a variable `SDC-

chipset', which would be used throughout the driver when distinctions must be

made.  See the Trident and PVGA1/WD drivers for examples (the Tseng ET3000

and ET4000 are counter-examples - these were implemented before the driver

interface allowed for multiple chipsets, so this example should NOT be fol-

lowed).  Note that you should only distinguish versions when your driver

needs to do things differently for them.  For example, suppose the SDC driver

supports the SDC-1a, SDC-1b, and SDC-2 chipsets.  The -1a and -1b are essen-

tially the same, but different from the -2 chipset.  Your driver should sup-

port the -1 and -2 chipsets, and not distinguish between the -1a and -1b.

This will simplify things for the end user.

In cases where you want to give the user control of driver behavior, or there

are things that cannot be determined without user intervention, you should

use ``option'' flags.  Say that board vendors that use the SDC chipsets have

the option of providing 8 or 16 clocks.  There's no way you can determine

this from the chipset probe, so you provide an option flag to let the user

select the behavior from the XF86Config file.  The option flags are defined

in the file ``xf86_option.h''.  You should look to see if there is already a

flag that can be reused.  If so, use it in your driver.  If not, add a new

#define, and define the string->symbol mapping in the table in that file.  To

see how option flags are used, look at the ET4000, PVGA1/WD, and Trident

drivers.

6.2  Data Structures

Once you have an understanding of what is needed from the above description,

it is time to fill in the driver data structures.  First we will deal with

the `vgaSDCRec' structure.  This data structure is the driver-local structure

that holds the SVGA state information.  The first entry in this data struc-

ture is ALWAYS `vgaHWRec std'.  This piece holds the generic VGA portion of

the information.  After that, you will have one `unsigned char' field for

each register that will be manipulated by your driver.  That's all there is

to this data structure.

Next you must initialize the `SDC' structure (type `vgaVideoChipRec').  This

is the global structure that identifies your driver to the server.  Its name

MUST be `SDC', in all caps - i.e. it must match the directory name for your

driver.  This is required so that the Link Kit reconfiguration can identify

all of the requisite directories and global data structures.

The first section of this structure simply holds pointers to the driver func-

tions.

Next, you must initialize the information about how your chipset does bank

switching.  The following fields must be filled in:

1.          ChipMapSize - the amount of memory that must be mapped into the

server's address space.  This is almost always 64k (from 0xA0000 to

0xAFFFF).  Some chipsets use a 128k map (from 0xA0000 to 0xBFFFF).  If

your chipset gives an option, use the 64k window, as a 128k window

rules out using a Hercules or Monochrome Display Adapter card with the

SVGA.

2.          ChipSegmentSize - the size of each bank within the ChipMapSize

window.  This is usually also 64k, however, some chipsets split the

mapped window into a read portion and a write portion (for example the

PVGA1/Western Digital chipsets).

3.          ChipSegmentShift - the number of bits by which an address will

be shifted right to mask of the bank number.  This is log-base-2 of

ChipSegmentSize.

4.          ChipSegmentMask - a bitmask used to mask off the address within

a given bank.  This is (ChipSegmentSize-1).

5.          ChipReadBottom,ChipReadTop - the addresses within the mapped

window in which read operations can be done.  Usually 0, and 64k,

respectively, except for those chipset that have separate read and

write windows.

6.          ChipWriteBottom,ChipWriteTop - same as above, for write opera-

tions.

7.          ChipUse2Banks - a boolean value for whether this chipset has

one or two bank registers.  This is used to set up the screen-to-screen

operations properly.

There are three more fields that must be filled in:

1.          ChipInterlaceType - this is either VGA_NO_DIVIDE_VERT or

VGA_DIVIDE_VERT.  Some chipsets require that the vertical timing num-

bers be divided in half for interlaced modes.  Setting this flag will

take care of that.

2.          ChipOptionFlags - this should always be `{0,}' in the data

structure initialization.  This is a bitfield that contains the Option

flags that are valid for this driver.  The appropriate bits are ini-

tialized at the end of the Probe function.

3.          ChipRounding - this gets set to the multiple by which the vir-

tual width of the display must be rounded for the 256-color server.

This value is usually 8, but may be 4 or 16 for some chipsets.

6.3  The Ident() function

The Ident() function is a very simple function.  The server will call this

function repeatedly, until a NULL is returned, when printing out the list of

configured drivers.  The Ident() function should return a chipset name for a

supported chipset.  The function is passed a number which increments from 0

on each iteration.

6.4  The ClockSelect() function

The ClockSelect() function is used during clock probing (i.e. when no

`Clocks' line is specified in the XF86Config file) to select the dot-clock

indicated by the number passed in the parameter.  The function should set the

chipset's clock-select bits according to the passed-in number.  Two dummy

values will be passed in as well (CLK_REG_SAVE, CLK_SAVE_RESTORE).  When

CLK_REG_SAVE is passed, the function should save away copies of any registers

that will be modified during clock selection.  When CLK_REG_RESTORE is

passed, the function should restore these registers.  This ensure that the

clock-probing cannot corrupt registers.

This function should return FALSE if the passed-in index value is invalid or

if the clock can't be set for some reason.

6.5  The Probe() function

The Probe() function is perhaps the most important, and perhaps the least

intuitive function in the driver.  The Probe function is required to identify

the chipset independent of all other chipsets.  If the user has specified a

`Chipset' line in the XF86Config file, this is a simple string comparison

check.  Otherwise, you must use some other technique to figure out what

chipset is installed.  If you are lucky, the chipset will have an identifica-

tion mechanism (ident/version registers, etc), and this will be documented in

the databook.  Otherwise, you will have to determine some scheme, using the

reference materials listed below.

The identification is often done by looking for particular patterns in regis-

ter, or for the existence of certain extended registers.  Or with some

boards/chipsets, the requisite information can be obtained by reading the

BIOS for certain signature strings.  The best advise is to study the existing

probe functions, and use the reference documentation.  You must be certain

that your probe is non-destructive - if you modify a register, it must be

saved before, and restored after.

Once the chipset is successfully identified, the Probe() function must do

some other initializations:

1.          If the user has not specified the `VideoRam' parameter in the

XF86Config file, the amount of installed memory must be determined.

2.          If the user has not specified the `Clocks' parameter in the

XF86Config file, the values for the available dot-clocks must be deter-

mined.  This is done by calling the vgaGetClocks() function, and pass-

ing it the number of clocks available and a pointer to the ClockSe-

lect() function.

3.          It is recommended that the `maxClock' field of the server's

`vga256InfoRec' structure be filled in with the maximum dot-clock rate

allowed for this chipset (specified in KHz).  If this is not filled in

a probe time, a default (currently 90MHz) will be used.

4.          The `chipset' field of the server's `vga256InfoRec' structure

must be initialized to the name of the installed chipset.

5.          If the driver will be used with the monochrome server, the

`bankedMono' field of the server's `vga256InfoRec' structure must be

set to indicate whether the monochrome driver supports banking.

6.          If any option flags are used by this driver, the `ChipOption-

Flags' structure in the `vgaVideoChipRec' must be initialized with the

allowed option flags using the OFLG_SET() macro.

6.6  The EnterLeave() function

The EnterLeave() function is called whenever the virtual console on which the

server runs is entered or left (for OSs without virtual consoles, the func-

tion is called when the server starts and again when it exits).  The purpose

of this function is to enable and disable I/O permissions (for OSs where such

is required), and to unlock and relock access to ``protected'' registers that

the driver must manipulate.  It is a fairly trivial function, and can be

implemented by following the comments in the stub driver.

6.7  The Restore() function

The Restore() function is used for restoring a saved video state.  Note that

`restore' is a bit of a misnomer - this function is used to both restore a

saved state and to install a new one created by the server.  The Restore()

function must complete the following actions:

1.          Ensure that Bank 0 is selected, and that any other state infor-

mation required prior to writing out a new state has been set up.

2.          Call vgaHWRestore() to restore the generic VGA portion of the

state information.  This function is in the vgaHW.c file.

3.          Restore the chipset-specific portion of the state information.

This may be done by simply writing out the register, or by doing a

read/modify/write cycle if only certain bits are to be modified.  Be

sure to note the comment in the sample driver about how to handle

clock-select bits.

6.8  The Save() function

The Save() function is used to extract the initial video state information

when the server starts.  The Save() function must complete the following

actions:

1.          Ensure that Bank 0 is selected.

2.          Call vgaHWSave() to extract the generic VGA portion of the

state information.  This function is in the vgaHW.c file.

3.          Extract the chipset-specific portion of the state information.

6.9  The Init() function

The Init() function is the second most important function in the driver

(after the Probe() function).  It is used to initialize a data structure for

each of the defined display modes in the server.  This function is required

to initialize the entire `vgaSDCRec' data structure with the information

needed to put the SVGA chipset into the required state.  The generic VGA por-

tion of the structure is initialized with a call to vgaHWInit() (also located

in vgaHW.c).

Once the generic portion is initialized, the Init() function can override any

of the generic register initialization, if necessary.  All of the other

fields are filled in with the correct initialization.  The information about

the particular mode being initialized is passed in the `mode' parameter, a

pointer to a `DisplayModeRec' structure.  This can be dereferenced to deter-

mine the needed parameters.

If you only know how to initialize certain bits of the register, do that

here, and make sure that the Restore() function does a read/modify/write to

only manipulate those bits.  Again, refer to the existing drivers for exam-

ples of what happens in this function.

6.10  The Adjust() function

The Adjust() function is another fairly basic function.  It is called when-

ever the server needs to adjust the start of the displayed part of the video

memory, due to scrolling of the virtual screen or when changing the displayed

resolution.  All it does is set the starting address on the chipset to match

the specified coordinate.  Follow the comments in the stub driver for details

on how to implement it.

6.11  The ValidMode() function

The ValidMode() function is required.  It is used to check for any chipset-

dependent reasons why a graphics mode might not be valid.  It gets called by

higher levels of the code after the Probe() stage.  In many cases no special

checking will be required and this function will simply return TRUE always.

6.12  The SaveScreen() function

The SaveScreen() function is not needed by most chipsets.  This function

would only be required if the extended registers that your driver needs will

be modified when a synchronous reset is performed on the SVGA chipset (your

databook should tell you this).  If you do NOT need this function, simply

don't define it, and put `NoopDDA' in its place in the vgaVideoChipRec struc-

ture initialization (NoopDDA is a generic-use empty function).

If you DO need this function, it is fairly simple to do.  It will be called

twice - once before the reset, and again after.  It will be passed a parame-

ter of SS_START in the former case, and SS_FINISH in the latter.  All that

needs to be done is to save any registers that will be affected by the reset

into static variables on the SS_START call, and then restore them on the

SS_FINISH call.

6.13  The GetMode() function

The GetMode() function is not used as of XFree86 1.3; its place in the

vgaVideoChipRec should be initialized to `NoopDDA'.

At some point in the future, this function will be used to enable the server

and/or a standalone program using the server's driver libraries to do inter-

active video mode adjustments.  This function will read the SVGA registers

and fill in a DisplayModeRec structure with the current video mode.

6.14  The FbInit() function

The FbInit() function is required for drivers with accelerated graphics sup-

port.  It is used to replace default cfb.banked functions with accelerated

chip-specific versions.  vga256LowlevFuncs is a struct containing a list of

functions which can be replaced.  This struct defined in vga256.h.  Examples

of FbInit() functions can be found in the et4000, pvga1 and cirrus drivers.

If you do NOT need this function, simply don't define it, and put `NoopDDA'

in its place in the vgaVideoChipRec structure initialization.

7.  Building The New Server

As in the setup work, the steps for building the server depend whether you

are working in the source tree or in the Link Kit.  Here are the steps for

the initial build after installing your new driver files:

o If you are working in the source tree, follow these steps:

Go to xc/programs/Xserver, and enter `make Makefile', then `make Make-

files depend all'

o If you are working in the Link Kit, follow these steps:

1.  Go to /usr/X11R6/lib/Server, and enter `./mkmf'

2.  In the same directory, enter `make'

To rebuild the server after the initial build (e.g. after making changes to

your driver):

o      If you are working in the source tree, follow these steps:

1.       Go to the appropriate drivers/ directory (e.g., xc/pro-

grams/Xserver/hw/xfree86/vga256/drivers), and enter `make'.

2.       Go to xc/programs/Xserver, and enter `make loadXF86_SVGA' (to

link the color server), `make loadXF86_VGA16' (to link the 16

color server) or `make loadXF86_Mono' (to link the mono server).

o      If you are working in the Link Kit, follow these steps:

1.       Go to the appropriate driver directory, and enter `make'.

2.       Go to /usr/X11R6/lib/server, and enter `make loadXF86_SVGA'

(to link the color server) or `make loadXF86_VGA16' (to link the

16 color server) or `make loadXF86_Mono' (to link the mono

server).

8.  Debugging

Debugging a new driver can be a painful experience, unfortunately.  It is

likely that incorrect programming of the SVGA chipset can lock up your

machine.  More likely, however, is that the display will be lost, potentially

requiring a reboot to correct.  It is HIGHLY recommended that the server be

run from an attached terminal or a network login.  This is the only rational

way in which a debugger can be used on the server.  Attempting to use multi-

ple VTs for debugging is basically a waste of time.

Because of the potential for locking up the machine, it is a VERY good idea

to remember to do a `sync' or two before starting the server.  In addition,

any unnecessary filesystems should be unmounted while the debugging session

is going on (to avoid having to run unnecessary fsck's).

By default the server is built without debugging symbols.  The server can

grow VERY large with debugging enabled.  It is very simple to rebuild your

driver for debugging, though.  Do the following:

1.  Go to the driver directory.

2.  Edit the Makefile.  Look for the SECOND definition of `CDEBUGFLAGS'.

Change this definition to

CDEBUGFLAGS = -g -DNO_INLINE

(this will enable debugging symbols and disable inlining of functions,

which can make single-stepping a nightmare).

3.  Remove the `sdc_driver.o' file.

4.  Now follow the steps above for rebuilding the server.

(Alternatively, instead of editing the Makefile, you can simply do

`make CDEBUGFLAGS="-g -DNO_INLINE"' after removing the old .o file,

then rebuild the server as described above).

This will give you a server with which you can set breakpoints in the driver

functions and single-step them.  If you are working in the source tree, and

just learning about SVGA programming, it may be useful to rebuild vgaHW.c

with debugging as well.

9.  Advice

I cannot stress this enough - study all available references, and the exist-

ing code, until you understand what is happening.  Do this BEFORE you begin

writing a driver.  This will save you a massive amount of headache.  Try to

find a driver for a chipset that is similar to yours, if possible.  Use this

as an example, and perhaps derive your driver from it.

Do not let the gloom-and-doom in the debugging section  discourage you.

While you will probably have problems initially (I still do), careful, delib-

erate debugging steps can bear fruit very quickly.  It is likely that, given

a good understanding of the chipset, a driver can be written and debugged in

a day or two.  For someone just learning about this kind of programming, a

week is more reasonable.

10.  Advanced Topics

Newer chipsets are getting into two advanced areas: programmable clock gener-

ators, and accelerated capabilities (BitBlt, line drawing, HW cursor).  These

are new areas, and the formal interfaces to them are not yet defined.  It is

advised that you contact the XFree86 team and get involved with the develop-

ment/beta-testing team if you need to be working in these areas.

11.  References

o Programmer's Guide to the EGA and VGA Cards, 3rd ed.

Richard Ferraro

Addison-Wesley, 1994

ISBN 0-201-62490-7

(This is the bible of SVGA programming - it has a few errors, so watch

out.  The third edition also covers several accelerated video cards.)

o vgadoc4.zip

Finn Thoegersen

(This is a collection of SVGA and other chipset documentation.  It is

available on most MS-DOS/Windows related FTP archives, including

wuarchive.  It is DOS/BIOS oriented, but is still extremely useful,

especially for developing probe functions.)

12.  Vendor Contact Information

ATI Technologies (VGA-Wonder, Mach8, Mach32, Mach64)

33 Commerce Valley Drive East" Thornhill, Ontario

Canada L3T 7N6

(905) 882-2600 (sales)

(905) 882-2626 (tech support)

(905) 764-9404 (BBS)

(905) 882-0546 (fax)

Chips & Technologies

???

Cirrus Logic (SVGA, Accelerators - CL-GD5426)

3100 West Warren Ave.

Fremont, CA  94538

(510) 623-8300 (N. CA, USA)

(49) 8152-40084 (Germany)

(44) 0727-872424 (UK)

Genoa Systems (GVGA)

75 E. Trimble Road

San Jose, CA 95131

(408) 432-9090 (sales)

(408) 432-8324 (tech support)

Headland Technologies, Inc (Video-7 VGA 1024i, VRAM II)

46221 Landing Parkway

Fremont, CA  94538

(415) 623-7857

Oak Technology, Inc (OTI-067,OTI-077)

139 Kifer Ct.

Sunnyvale, CA 94086

(408) 737-0888

(408) 737-3838 (fax)

S3 (911, 924, 801/805, 928, 864, 868, 964, 968, 764, 765)

(408) 980-5400

Trident Microsystems Inc (8800, 8900, 9000)

205 Ravendale Dr

Mountainside, CA 94043

(415) 691-9211

Tseng Labs Inc,

6 Terry Drive

Newtown, PA  18940

(215) 968-0502

Weitek (Power9000, 5186)

1060 E. Arques Ave,

Sunnyvale, CA  94086

(408) 738-5765

Western Digital

(714) 932-4900

Generated from XFree86: xc/programs/Xserver/hw/xfree86/doc/sgml/VGADriv.sgml,v 3.13 1997/01/25 03:22:16 dawes Exp $

$XConsortium: VGADriv.sgml /main/9 1996/10/28 05:13:22 kaleb $

$XFree86: xc/programs/Xserver/hw/xfree86/VGADriverDoc/VGADriver.Doc,v 3.27.2.5 1999/06/25 08:56:25 hohndel Exp $