SLX_RzK                                                12/08/1994

Remote access of an Ethernet can be well done with SLIP, as long as

the Internet Protocol (IP) is used. Even higher baudrates (38400,

57600 and 115200) works fine with a good PC, modem and well

functioning serial card.

The demand for Remote Access to Networks is increasing. ISDN brings

more speed to serial lines, but people want to be able to use

other protocols too. Thats the reason we introduced the "SLX", which is

very close to SLIP.

SLX sends not only the IP-Part of the frame like SLIP, instead it sends

the whole Ethernet frame including address- and type-field across

the serial line.

With SLX you can use nearly every Network-Software that can work with

packet drivers. If you additionally load PDIPX or PDETHER, you can

use all Novell-Software. Currently we do not know if there is a

PKT- to NDIS converter, but this will be a matter of time.

If you have only two Stations, you can even eleminate the need of

a network board. With modems or ISDN Terminal Adapters the distance between

both stations can be as long as you want.

A Station that can handle two Network-Interfaces can be used as a

remote assess node.

The remote access node, we produce is called "Ethernet Box" and is

commercially available. The Ethernet Box can handle a modem (or

Terminal Adapter for ISDN) and several protocols. It is possible to

use it as a Telnet-Terminal-Server, as SLIP-Gateway, as SLX-Gateway,

as remote Bridge (if the caller is another Ethernet Box in Bridge mode),

or as a Network Analyzer giving various statistics, watch with filters

and Ethernet-testing. The Box can be serviced via Telnet and SNMP.

As in SLX-Mode the whole frame is coming over the serial line, the

PC has not only to provide a protocol address, but an unique

Ethernet-Hardware-Address too. The current driver sets the hardware address

to 00:00:FB:AC:00:00, but it should be overwritten with the program

PKTADDR by an unique address.

There is an example of a batch file SLX.BAT which is using the utility

programs AT, DCD, RING ant DTR. It diales into an Ethernet Box, can use

the dial-back feature, and starts a Network-Program.

Installation on a PC using COM1/2:

----------------------------------

Loading of SLX-Driver.

SLX_RzK 96 -h 4 0x3f8 38400

PKTADDR 96 00:00:FB:AC:xx:yy      ; take a Adresse of your own!

; You can buy one from RzK.

In case of WINDOWS 3.x give an extra:

WINPKT

to load the PKTDRVR Windows interface. In this case

you must also disable the used com-port for Windows.

Then you can load your Network-Software:

PDIPX

If you want to dial a serial connection wit a modem, you can do this with

the small utility AT. The DTR-Signal should be on in this case.

AT COM1: DP89764321

With the DCD-Utility you can wait until DCD is on:

DCD COM1:

At the end of the Connection you can drop DTR to disconnect:

DTR COM1: OFF

An example of a batch-procedure to handle a connection automaticly is

included in the file SLX.BAT.

RzK Doris Koepke                           Tel:  49 2683 4547

Postfach 1200                              Fax:  49 2683 4537

D-53563 Asbach Ww.                         Mail: 70126.1660@compuserve.com

Germany