Telix HST Service Script Release 1.0

HST MODEM SETUP UNDER Telix 3.11

Setting up your modem for Telix use is quite simple.  In the Telix

configuratin (ALT_O), make sure that the following parameters are

specified:

MODEM & DIALING OPTIONS

Init String:         ATZ~~~AT&B1^M

Hang Up String:      ~~~+++~~~ATH0^M

Dial abort string:   ^M

Drop DTR to Hang Up: on

Note: Use of the ~ as delay character and ^M as carriage return

will insure proper script operation under Telix release 3.11.

TERMINAL OPTIONS

Baud:         19200

XON/XOFF:     off

CTS/RTS:      on

DSR/DTR:      off

DIALING DIRECTORY ENTRIES

Set the baud rate, for dialing directory entries that support another

USR HST modem at the other end, to 19200 baud for the best speed.

That's it.  proper switch settings are:

1,2,6,7,10: UP

All others: DOWN

Andrzej Taramina

58 Aquila Court

Rexdale, Ontario

Canada, M9W 5J2

12/13/1988