ERSATZ-11 BETA VERSION 1.1

PDP-11 SYSTEM SIMULATOR

Release date: 22-Mar-1995

Copyright (C) 1994, 1995 by John Wilson

All rights reserved

E11.EXE is a beta test version of a full system emulator

for the PDP-11.  It runs on any AT-class machine with an 80186

or better CPU, and requires around 350KB of free memory.  It is

intended to boot and run any PDP-11 operating system (it has

been tested with RT-11 (all flavors), RSX-11M, RSTS/E, IAS,

2.9BSD UNIX, Fuzzball, and XXDP+;  it seems to have some

problems with VM under TSX+, and no attempt has been made to

test DOS/BATCH, MUMPS, or any flavor of UNIX other than 2.9BSD;

2.11BSD will definitely not work due to its need for "cadillac"

hardware).

Emulated configuration:

* PDP-11/34a CPU with selectable extensions

* FP11A floating point processor

* 248KB main memory

* RL11/RL01-02 disk drives (up to 4)

* RK611/RK06-07 disk drives (up to 8)

* RX211/RX02 or RX11/RX01 8" floppy (using image files and/or

1.2MB floppies)

* DL11 terminal ports;  console and up to 15 others (VT100,

and/or uses COM ports and/or LPT ports)

* LP11 printer ports;  up to 4 (same devices as DL11)

* PC11 paper tape reader/punch (using files)

* DELUA ethernet ports;  up to 4 (using packet drivers)

* KW11L line clock (50/60 Hz, settable)

* display register (using special hardware)

Ersatz-11 is available by anonymous FTP from TATS.WIZVAX.NET, or

perhaps TATS.WIZVAX.COM depending on how screwed up the name

server is when you try, anyway it's 199.181.141.53.  The

directory is pub/e11, filenames e11.exe, e11.hlp, and e11.doc.

The emulator speed depends on the application (changing MMU

registers is an expensive operation so multiuser OSes tend to

run more slowly than RT-11FB does, for example), but in general

it runs noticeably above the speed of a real PDP-11/34a (with

cache) on a 486-DX2/80.  Your mileage may vary.  It has

successfully booted and run RT-11FB on a 4.77 MHz IBM PC with a

V20 processor (80186-compatible), but it barely stayed ahead of

the clock interrupts.

The FP11 emulation requires a math coprocessor.  If the PC

has none, then the emulated PDP-11 will have no FPP either.

Intel Pentium CPUs that have the floating point divider bug are

detected and a workaround is used to get correct results at a

slight speed penalty (for DIVF/DIVD only).

I consider this program to be a beta test release because

there are still some reported bugs that I've been unable to

duplicate, and there are many unfinished or unstarted features

that I plan to add, most of which hinge on porting the program

to run in 80386 protected mode so as to be able to implement

22-bit addressing efficiently.  My apologies to anyone who wants

to run it on an 80286 or earlier, the final version will most

likely require a 386.  Besides, the program is growing so

rapidly that nothing has been left untouched long enough to be

considered 100% tested.  However in my own experience the

emulator has been more reliable than real PDP-11s.

Planned additions:

* MSCP disks.  Don't hold your breath, but it will happen

eventually in some form.  Can anyone tell me the interleave

scheme (if any) used on RX23 and RX33 disks?

* Some kind of tape support.  ASPI, QIC-80, FILETAPE, or maybe

something else.

The system has been tested under the XXDP+ diagnostic

monitor.  It passes the KD11EA diagnostics DFKAA, DFKAB, and

DFKAC, and the FP11A diagnostics DFFPA, DFFPB, and DFFPC.  It

fails all MMU diagnostics due to the absence of the maintenance

mode.

COMMANDS

Ersatz-11 recognizes a number of keyboard commands.  These

are entered at the "E11>" prompt, which appears when the PDP-11

is halted but may be brought up at any time by pressing

Alt-SysReq (or by pressing the BREAK key on a serial terminal if

the console terminal (TT0:)  has been ASSIGNed to a COM port).

Commands (and parameters and switches) may generally be

shortened to any unique abbreviation.

INITIALIZATION FILE

When E11 is first started, it looks for a file named

"E11.INI" first in the current directory, and then in the

directory where E11.EXE is located (or, it searches the PATH if

you're still running DOS V2.X).  If this file exists than a

command is read from the file each time E11 would otherwise

prompt for a user command;  input for the console DL11 still

comes from the keyboard, so the init file may be used both to

start up the system and to shut it down (E11 will continue after

the BOOT command if you type Alt-SysReq).  If a line's first

non-blank character is ';' or '!', it is treated as a comment.

Processing of the init file can be disabled by starting E11 with

the "/NOINIT" switch, and a different file may be specified

using the "/INIT:file" switch;  the default extension is ".INI"

and the above search rules apply unless the filename contains a

drive or pathname specification.

Typical E11.INI file:

mount dl0: rt11.dsk

; uncomment the ASSIGN command to use a COM port for console I/O

; (baud rate must have been initialized from DOS using a MODE command)

;assign kb0: com3: /irq5

boot/rt11 dl0:

; control returns to the next line when the user presses Alt-SysRq

quit

COMMAND DESCRIPTIONS

======= ============

@file

Accepts input from the specified file as if it had been typed at

the E11 prompt.  The default extension is ".CMD", and search

rules are the same as for the E11.INI initialization file.

Lines read from the file are not echoed, and indirect files may

not be nested.

ASSIGN ddu:  Fn

ASSIGN ddu:  COMn:  [/IRQn [/SHARE]]

ASSIGN ddu:  LPTn:  [/IRQn [/SHARE]]

Assigns a physical PC device to emulate a particular DL11

(TT0:-TT15:)  or LP11 port (LP0:-LP3:).  The first (and possibly

only) argument after the PDP-11 device name may be either the

name of a function key F1-F12, or a serial port COM1:-COM4:, or

a line printer port LPT1:-LPT4:.

If a function key is given, then the specified port is connected

to a simulated VT100 which can be put up on the screen by

pressing Alt and the specified function key (note that the

screens assigned to F11 and F12 are not accessible if you still

have the 84-key AT keyboard).  When one screen is being

displayed on the PC screen, the others (up to 11) are maintained

invisibly in memory, so they will be up to date when you switch

the display to them by pressing Alt and the function key for the

screen you want.

UNTESTED FEATURE:  If there are two video adapters on the PC

(e.g.  an SVGA and a Hercules mono card), then one DL11/LP11

pseudo VT100 may be displayed on each.  The Alt-function keys

choose which of the 12 possible screens is displayed on the

primary monitor, and the Ctrl-function keys choose which is on

the secondary monitor.  Note that it is not possible to display

the same port on both monitors at once.  I no longer have such a

setup so the code is untested.

If the name of a COM port is given, then the specified DL11/LP11

port is connected to that port.  Any IRQ from IRQ0 to IRQ15 may

be specified, or if the IRQ is omitted then IRQ4 is used for COM

ports whose I/O base address is 300h or more, IRQ3 for 2FFh or

less.  These defaults have been in use since XTs were the

standard, and most multi I/O boards will be set up this way.

However it's not uncommon with newer boards for COM3 to use IRQ5

and COM4 to use IRQ2 (or IRQ9 really, which is effectively the

same on an AT);  you will have to use the /IRQ5 and /IRQ2

switches in this case.  The /SHARE switch (the /IRQn switch is

required if /SHARE is given) means that the serial port has

special IRQ sharing hardware (as documented in the IBM AT

Technical Reference Manual) so that more than one device may use

the same IRQ at once;  this hardware is relatively rare, so if

you don't know whether you have it, you probably don't.  Note

that without this hardware it's not possible for more than one

device to be actively using the same IRQ at the same time;  so

for example if you have a mouse attached to COM1 using IRQ4 and

it has been initialized by MOUSE.COM, E11 will not be able to

use COM3 if it also uses IRQ4 (an error message will be given if

you try).

The baud rate, number of data bits, etc.  for a COM port should

be set with a MODE command from DOS before Ersatz-11 is run.

If the name of an LPT port is given, then the specified

DL11/LP11 port is connected to that port.  /IRQn and /SHARE may

be given as for COM ports;  the default IRQ for all LPT ports is

IRQ7.  E11's LPT handler uses interrupts;  I've seen vague

references to interrupts not working correctly with some LPT

ports, or maybe it was with some printers, but having polled I/O

as an option would be horribly slow (either printing speed or

CPU emulation or both would suffer) so it's not supported.

Interrupts work great with my brand-X LPT port and 13-year-old

NEC Spinwriter.  Trouble is most likely if you have multiple LPT

ports, since they may all try to drive IRQ7 at once.

LPT output is passed transparently, so you'll need to make sure

that your OS and printer agree on whether lines end in <CRLF> or

just <LF>, and on whose responsibility it is to interpret tabs

and form feeds.

Note that it is possible to ASSIGN a TT:  port to a printer, or

to ASSIGN an LP:  port to a screen.  Why you'd want to do this I

don't know but who am I to stop you.  The reason for making them

both use the same pool of devices was so that they could both

access COM ports, since serial terminals and serial printers are

both reasonable devices.  LP:  ports attached to COM ports or

screens respond to XON/XOFF flow control.

The ASSIGN command fails if the specified COM or LPT port

doesn't exist, or if the specified (or default) IRQ is already

in use and the /SHARE switch is not given, or if you're trying

to steal TT0:'s device for some other port (there must always be

a TT0:  since that's E11's console terminal).

ASSIGN XEn:  PKTD[=vv] [proto1 proto2 proto3 ...]

Assigns a packet driver to emulate the specified DELUA ethernet

port.  If specified, "vv" is the hexadecimal interrupt vector to

which the packet driver is attached.  If the interrupt vector is

not given, then the range of vectors from 20 to FF is searched

until a packet driver is found that isn't already in use by

another simulated ethernet port.  A list of hex 16-bit DIX

protocol numbers, up to ten total, may be specified, in which

case E11 asks the packet driver to pass only frames of those

types.  This may allow E11 to coexist with other protocol stacks

running on the same PC, as long as they each use different sets

of protocol numbers and neither one needs to change the station

address after the other is loaded.

DECnet requires the ability to change the address (it wants the

station address to be based on the protocol address to avoid the

additional overhead of performing address resolution over the

network, as TCP/IP does), so in general running DECnet under E11

will require either that no other network software be running on

the PC, or that you install a second ethernet board for E11's

exclusive use.  This is because the packet driver refuses to

change the station address once the board is already in use so

as not to surprise protocol stacks that were already running

when the change was made and thought they knew the address.

Freeware packet drivers are available via anonymous FTP from

many sources including FTP.FTP.COM.  Ersatz-11 requires packet

drivers that conform to version 1.10 or later of the packet

driver specification as published by FTP Software.

BOOT ddu:  [/switches]

Boots the system from the specified disk.  The disk must have

been mounted with the MOUNT command.  The optional switch is an

OS name;  for now the only meaningful ones are /RT11 and /RSTS,

/RSX is defined too but has no effect.  This has to do with the

method used to pass time and date information to a newly booted

monitor.  RT-11 ignores the time and date passed at 005000

unless the NOP in the first word of the bootstrap is cleared to

0 (HALT) and the bootstrap is entered at 000002.  RSTS believes

the time and date at 001000 (in a different format from RT-11)

regardless of whether its NOP was nuked, but later versions of

RSTS save the first word of the bootstrap and execute it later

(anyone know why?), so they will halt if the system was booted

the RT-11 way.  Hence the need for the switch.  Note that the OS

switches are meaningful only on block devices.  If you like

typing the time and date manually (or your PC has no RTC) then

don't worry about the switch.  I don't know enough about RSX or

any other OS to set up the time/date for it, if that's even

possible;  however I am planning to add emulation of the

PDP-11/94 TOY clock if I can figure it out, then a privileged

program could read it out and set the RSX clock.

There is also a /HALT switch, which means to go as far as

loading block 0 into core and setting up the registers, but to

stop there.  This can be handy for debugging boot blocks.

The BOOT PR:  command expects a tape image in absolute binary

format, as produced by the "LINK /LDA" command under RT-11.

CALCULATE expr

(& expr is a synonym)

Calculates the value of a 32-bit octal expression and displays

the result in octal, decimal, ASCII and radix-50.  The operators

are */+-, unary +-, and (), with the usual precedence.  Numbers

are either octal digit strings, or decimal if they contain 8 or

9 or end in ".", or hex if preceded by "^X", or radix-50

triplets if preceded by "^R";  or general register contents may

be specified using the names R0-R5 (with a "'" suffix to

indicate the other register set) or SP or PC, R$ or PS means the

process status word, and something of the form "'a" means the

ASCII value of the character "a".

DEASSIGN ddu:

Disables the specified DL11 port (TTn:), LP11 port (LPn:), or

DELUA port (XEn:).  Deassigning TT0:  is not allowed (either

explicitly, or implicitly by ASSIGNing its PC device to another

PDP-11 device).

DEPOSIT addr val

Deposits the word <val> at absolute 18-bit address <addr>, which

is forced even.  An error message is returned if an attempt is

made to access a nonexistent CSR in the I/O page.

EXAMINE addr

Examines the word at absolute 18-bit address <addr>, which is

forced even.  An error message is returned if an attempt is made

to access a nonexistent CSR in the I/O page.

FPREGISTER [r v1 v2 [v3 v4]]

Sets or displays the FPP registers.  r is the FP accumulator

number, 0-5, and v1-v4 are two or four 16-bit octal words to

write in the register (sorry, not decimal).  If no arguments are

given then the octal contents of all six ACs are given, along

with octal displays of the FPS, FEC, and FEA, and also a

bit-by-bit display of FPS.

GO [addr]

Starts the machine at the specified address, or at the address

currently in the program counter if none is given.

HALT

If the machine is running, halts it and displays the registers.

Otherwise a no op.

HELP [command]

Explains use of Ersatz-11 commands.  Just type "HELP" for a

list.

INITIALIZE

Initializes all emulated I/O devices, disables the MMU, sets the

CPU mode to "kernel."

LIST [addr]

Disassembles eight instructions starting at the specified 16-bit

address (within the current I-space map) if it is given, or

otherwise at the first address following the last one

disassembled by the most recent LIST or REGISTER command.

LOG ddu:  [filename] [/APPEND]

Logs all output to the specified character device (TTn:  or

LPn:)  in the specified file.  If no filename is specified, any

existing log file for that device is closed.  The /APPEND switch

means to append to an existing log file, rather than replacing

it.

LOG DLn:  [filename] [/APPEND]

LOG DMn:  [filename] [/APPEND]

LOG DXn:  [filename] [/APPEND]

LOG HDn:  [filename] [/APPEND]

Logs commands sent to the RL11, RK611, or RX[2]11 controller, or

the HD_SYS.EXE pseudo-controller, to the specified file.  If no

file is specified, the current log file, if any, is closed.  The

unit number is insignificant, all commands to the controller are

logged regardless of the currently selected unit.  The /APPEND

switch means to append to an existing log file, rather than

replacing it.

LOG XEn:  [filename] [switches]

Controls logging of ethernet events.  If a filename is specified

then the log file is opened.  If switches are specified they

specify what events are to be logged;  "/[NO]COMMANDS" controls

logging of port commands, "/[NO]RECEIVE" controls logging of

received frames, and "/[NO]TRANSMIT" controls logging of

transmitted frames.  The switches may be specified when the log

file is first opened, or later in LOG commands with the filename

parameter missing to change what is being logged without having

to open a new log file.  If neither the filename nor any

switches are specified, any existing log file for that device is

closed.  If no switches are specified when the file is first

opened, the default is "/RECEIVE /TRANSMIT".  In addition, the

/APPEND switch means to append to an existing log file, rather

than replacing it.

MOUNT devu:  path/filename.ext [/switches]

Mounts the specified DOS file on the specified PDP-11 disk drive

(DL0:-DL3:, DM0:-DM7:, DX0:-DX1:, HD0:-HD7:);  that is, PDP-11

reads and writes to that drive access the specified file, which

is a byte-by-byte image of a disk, which you presumably loaded

from a real PDP-11 using Kermit or DECnet or some equivalent, or

built using FLX.EXE or RT11.EXE or PUTR.COM.

For DL:, the drive is set up to emulate an RL01 or RL02

depending on the size of the file;  each drive is set

separately, so you can have a mix of RL01s and RL02s.  The

/RONLY switch has the same effect as pressing the WRITE PROT

button on a real drive (/WPROTECT is a synonym).

For DM:  things are similar to DL:, each of the 8 drives can be

either an RK06 or RK07 depending on the file size.  /RONLY works

as above.

For DX:  (DY:  is a synonym), the drive is set up to emulate an

RX01, RX02, or RX03 disk depending on the size of the file or

whether the /SSSD, /SSDD etc.  switch was given.  The file size

is also used to determine whether the file is a block-by-block

image of the PDP-11 floppy starting at logical block 0 (such as

one gotten by using Kermit to read the disk under a PDP-11 OS),

in which case the interleave needs to be un-done since the

PDP-11 device handler will re-do it;  or a sector-by-sector

image starting at track 0, side 0, sector 1.  The latter format

yields a slightly larger file because the PDP-11 floppy device

handlers skip track 0 (something to do with proposed ANSI

compatibility that never materialized as far as I know).

A second option for DY:  is to give the name of a 1.2MB floppy

drive instead of the name of a DOS image file, i.e.  A:  or B:.

Accesses to that RX02 unit will be redirected to the specified

floppy drive (and there's no need to re-MOUNT the drive when you

change disks) using a 26-sector format equivalent to that of a

real 8" disk drive.  In fact, if you somehow connect an 8" drive

to your PC floppy controller then it should be able to handle

RX01 media directly (I haven't had an opportunity to test this

though).  DEC's MFM format is weird so RX02 disks wouldn't be

compatible.  The RX211 "set density" command is implemented as a

full format, so you can format the weird 1.2MB disks using

FORMAT.SAV or SPEC%() or whatever you'd use on your real PDP-11.

Two problems:  (1) On a 1.2MB 5.25" drive the index hole is in

the same place for SS and DS media (8" drives use the index hole

to tell them apart) so E11 decides whether to report the disk as

double-sided by depending on the /SSxx or /DSxx switches to the

MOUNT command (/SSDD is the default);  the program detects disk

density changes like a real RX211 (well, more or less,

occasionally it takes a few tries to notice a change because the

PC controller doesn't have a "wrong density" error flag, it's

not that simple with IBM-style MFM) but still needs to be told

if the number of sides changes.  (2) Many, if not most, PC

floppy controllers have broken single density modes (the PC BIOS

is hard coded for double density so no one cares), so

FORMAT/SINGLE or anything else to do with actual single-density

media may not work on your PC.  FDC chips made by SMC are known

OK (FDC37C65LJP for one, the kind that takes one 16/32MHz xtal

and one 9.6MHz one), as are at least some by Goldstar.  In my

testing Intel's 82077 can't write SD disks but seems to be able

to read them with some retries;  according to the data sheets

for the NS PC8477B (which is supposed to be compatible) this is

because the chip requires an external pin (MFM) to be grounded

to enable SD mode, and for normal AT use it would be reasonable

to let it float high (required by RESET mode ID anyway) so I

imagine that's what most designers do.

An HD:  device exists for compatibility with the HD_SYS.EXE

hypothetical hard disk device in the demo version of the

so-called "Russian" PDP-11/03 emulator.  This way E11 can boot

RT11SJ images built for that program (you should type SET HZ 50

before booting them).  It supports up to eight units of

arbitrary sizes.  I have written an RT-11 device handler for

this device (not based on the Russian one, some extensions were

necessary for future compatibility with planned upgrades to E11)

with files up to 32MB, and Eduard Vopicka is working on one for

RSX-11M with a higher limit;  it is intended that this device

act as a stopgap for handling variable sized image files of

varying sizes until the MSCP emulation code is completed.  If

you want to write a driver for some other OS let me know and

I'll get around to documenting the device.

*** IMPORTANT NOTE ***

The simulator has no control over any caching of disk writes

that DOS may do, so it is important that you QUIT out of the

simulator to make sure all the pseudo-disk files get closed

properly, rather than simply switching the computer off, after

shutting down your PDP-11 OS.  Otherwise there is no guarantee

that all data written to disk by the PDP-11 has really made it

onto the DOS disk (in practice I've had no problems, but trouble

is theoretically possible so you've been warned).  But, the

RX211 1.2MB floppy emulation controls the hardware directly (in

fact it even intercepts INT 13h to keep DOS's fingers out of

things so you can't crash the machine by trying to MOUNT an

image file located on a DOS floppy while 1.2MB floppy emulation

is active) so as long as the PDP-11 is done with the disk it's

OK to take it out of the drive.

Also, the simulator has no control over "Green PC" BIOSes which

spin down hard drives after a specified period of inactivity.

When E11 accesses the drive after it's been shut down, the BIOS

will pause several seconds while it spins the drive back up.

During this period E11 is not running, so the simulated PDP-11

may drop incoming characters (but maybe not, E11 maintains a

small FIFO buffer on each line) and its clock will lose a few

seconds.  If this is a problem you may have to disable this BIOS

feature.  If anyone knows of a way for E11 to handle spinning

the drive up asynchronously after a timeout, without confusing

the BIOS, I'd love to hear about it, what little APM

documentation I have appears to be wrong, at least from my

testing (that and/or my BIOS is broken).  That way the PDP-11

could keep running and it would just see a slow disk transfer

once in a while.

MOUNT PP:  file

Mounts the specified file to receive output sent to the PC11

high speed paper tape punch.  The default extension is ".PAP".

MOUNT PR:  file

Mounts the specified file to provide input read from the PC11

high speed paper tape reader.  The default extension is ".PAP".

PROCEED [break]

Continues PDP-11 execution at the address currently in the

program counter.  If <break> is specified, then it is the

virtual address of a single hard breakpoint, where the PDP-11 is

guaranteed to stop if an instruction fetch is attempted starting

at that address, regardless of what mode the computer is

executing in, and regardless of whether the contents of that

location have changed since the breakpoint was set.  This can be

handy for tracing code that hasn't been loaded yet.  Note that

hard breakpoints and single stepping with the STEP command

interfere with the operation of the CPU T bit, so don't combine

them with a debugger (or CPU traps diagnostic program) running

on the PDP-11 or you'll get strange behavior.

QUIT

Exit the simulator, closing all image and log files and

resetting all devices that were in use.

REGISTER [r val]

If <r> and <val> are given, sets register <r> (0-7) in the

current register set to contain <val>.  Otherwise displays the

values of all eight registers, the condition codes, the current

and previous processor modes, and the current interrupt priority

level.  Note that the only way to set the PSW value is by

depositing it into location 777776.

SET CPU item [item...]

This command changes the emulated CPU type, either by changing

to a new model all at once, or on a feature-by-feature basis.

SET CPU 34A sets the emulation to be 11/34a compatible, SET CPU

35 sets it to be 11/35 compatible (mostly -- the 11/20-style

weirdness with MOV SP,-(SP) pushing the decremented SP, etc.  is

not emulated).  EIS and NOEIS tell whether the EIS instructions

exist, FPP and NOFPP turn the FPP emulation on or off (it's

permanently off if your PC has no 80x87), and MXPS and NOMXPS

enable/disable the MFPS and MTPS instructions.

There are many other items but they correspond to features that

either don't exist yet (so even though they appear in SHOW CPU

you get an error message when you try to turn them on), or

haven't had serious testing.  If you want to try them please let

me know how it turns out, I believe they work, but it wouldn't

be fair to claim I'm sure, that's why this is a beta release.

CPUERR    CPU error register.

CSM       CSM instruction (requires SUPMODE to work).

DSPACE    Split I/D space.

DUALREGSET  Dual register set.

MMTRAPS   11/45,55,70-style memory management traps, 3-bit ACF.

PIRQ      11/45-style 7-level software interrupts.

SPL       SPL instruction.

SUPMODE   Supervisor mode.

TSTSET    J-11 TSTSET, WRTLCK instructions.

This gives you the ability to roll your own CPU, which may not

correspond to any actual existing PDP-11 model.  This may annoy

your OS, and changing CPU types while running will almost

definitely startle it.  Anyway SHOW CPU will show you what

you've done.  The list of configurable features will grow in

future versions to include more actual PDP-11 models, this is

just what I have now.

SET DELAY device c1:n1 c2:n2 ...

Sets the number of instructions that the specified command

opcodes appear to take to complete on the indicated device.  The

device may be DELUA, DL11, LP11, PC11, RL11, RK611, RX11, or

RX211 (synonym for RX11).  There may be an arbitrary number of

parameters of the form "c:n" or "c=n", where c is the opcode for

the command and n is the number of PDP-11 instructions to delay

before signaling completion of the command.  Both numbers are

octal by default, but may be specified in decimal if they

contain the digits 8 or 9, or if they're terminated with "."

(actually any expression that would work with the CALCULATE

command works here).

The reason that device commands, such as "read sector" on an

RX02, or "transmit character" on a DL11, delay signaling

completion (by raising the "ready" flag and/or triggering an

interrupt) instead of completing right away (which would seem

natural in an emulator) is that some OS software contains

assumptions that at least a certain number of instructions are

guaranteed to be executed before a device is able to interrupt,

even when interrupts from that device are enabled.  The default

interrupt delays are set for the "worst case", so that each one

is long enough to avoid any known (or suspected) problems with

DEC OS software.  The SET DELAY command may be useful in cases

where your OS needs a longer delay than the default, or in cases

where your OS's treatment of a device is "clean" and you can

gain a noticeable I/O speed increase by setting all the delays

for that device to 1, or in cases where you're debugging OS

software and want to test against variety of interrupt rates.

If this means nothing to you, you can safely forget about it,

this command is intended mainly for advanced users.

Note that some devices don't have command opcodes per se, but

the SET DELAY command syntax requires one anyway for

consistency.  On DL11 SLUs, opcode "0" refers to the delay

between reading a character from the receiver buffer, and

getting the interrupt for the next character (only if it's the

second or later character of a function key sequence on an

emulated VT100, all other keyboard interrupts correspond to

actual asynchronous events);  and opcode "1" refers to the delay

between writing a character to the transmitter buffer, and

getting the completion interrupt (for emulated VT100s;  COM and

LPT ports use real completion interrupts).  Similarly, PC11

opcode "0" refers to how long it takes to read a character from

paper tape, and opcode "1" refers to how long it takes to write

one.  An LP11 has only one opcode, which is "0" and corresponds

to the same thing as opcode "1" of a DL11.  An RK611 has only

opcodes 0-17, but the SET DELAY command defines an opcode "20"

which refers to the delay between the interrupt that

acknowledges reception of a head movement command (which is

itself delayed), and the ATTN interrupt which signals completion

of the head movement.

SET DISPLAY NONE

SET DISPLAY PORT n

SET DISPLAY LPTn:

If PORT is specified, specifies the octal 80x86 I/O address of a

word port which when written as a word, sets the 16-bit display

register.  Building the trivial hardware to support this is left

as an exercise to the reader.

If a PC LPT port name is given, it specifies a port which has a

multiplexed LED board plugged into it, and E11 will refresh each

half on alternate 60Hz (50Hz) clock ticks;  there's a little

flicker but it works and requires no chips or power supply, just

build your board so that the D0-D7 lines (pins 2-9 of the DB25)

drive the anodes of the both the D0-7 and D8-15 LEDs (through

the same set of eight 100 ohm resistors since only one set of

LEDs will have their cathodes grounded at a time).  Then add two

NPN switching transistors (2N3904 etc.)  with the emitters

grounded (pin 25), the collectors connected to the cathodes of

all 8 LEDs for that byte, and the bases connected through a 1K

current limiting resistor to either /STROBE (pin 1) for the

D0-D7 LEDs or /INIT (pin 16) for the D8-D15 LEDs.  A bare PCB

board is available from the author at cost ($14.68 plus shipping

for the current batch, 12/94).

If NONE is specified, then the current DR value is available

only from the SHOW DISPLAY command (the default condition).

SET DISPLAY {DR | R0 | PC}

Sets what register is displayed on a hardware LED display

register (either the parallel port kind described above or the

kind that plugs into a bus slot and is addressed by a word OUT

instruction).  By default the DR is displayed (i.e.  the last

word written to 777570), but R0 or the PC may be selected

instead, since the null jobs in some operating systems display a

pattern in R0 (and the PC in some cases) during a WAIT

instruction.  The pattern may be used to get a rough idea of

system load, and the R0/WAIT method was a standard way to

display a number on the PDP-11/70.  For completeness, registers

R1-R5 or SP may be selected too.

SET FLOPPY RX[2]11

Sets the type of floppy controller that will be emulated.  This

command is necessary because the RX11 and the RX211 use the same

CSR and vector addresses, otherwise E11 could just emulate one

of each.  The default is RX211 (with a so-called "RX03" drive);

the RX11 is inferior in every way to the RX211 (and probably

doesn't work with your floppy controller anyway so you'll be

stuck with image files) so normally you wouldn't use this

command, but if you're trying to boot something that expects an

RX11 (e.g.  an RT-11 distribution with RX01 boot blocks) you'll

need this command since the RX211 is not software-compatible

with the RX11 (although it's very similar).

SET HERTZ {50 | 60}

SET HZ {50 | 60}

Sets the frequency of the KW11L line clock (startup default is

60);  Ersatz-11 reprograms the PC timer chip for this rate to

simulate line time clock interrupts, and then maintains a count

in software so that it knows when to trigger BIOS 18.2 Hz

interrupts;  there is some jitter due to the BIOS interval being

rounded down to the previous KW11L interrupt, but there is no

cumulative error so the DOS clock is still correct when you exit

out of E11.

SET KEYBOARD [NO]SWAP

SWAP sets the keyboard handler to exchange the functions of the

Caps Lock and left Ctrl keys for people who (understandably)

don't like the IBM Enhanced Keyboard.  NOSWAP sets the handler

back so that the keys work as marked.

SET [NO]SCOPE

Sets whether the console terminal is a scope or a hardcopy

terminal, for the purpose of handling rubout characters typed at

the "E11>" prompt.  Mainly useful if the console is redirected

to a COM port with a DECwriter (etc.)  plugged into it.  Also

determines whether typing ^L at the command prompt will attempt

to clear the screen.

SET SCROLL {HARD | SOFT}

Selects the mechanism used for video scrolling.  HARD scrolling

offers superior performance (it works by programming the video

board(s) to change the start address of the screen each time a

full-screen scroll is needed), but may expose problems in video

virtualization in GUI DOS boxes, or cause problems with TSRs

that use the display.  SOFT scrolling does things the slow

obvious way, by copying the whole screen a line up on each line

feed, and ought to work with anything.  HARD scrolling is the

default;  you should try putting SET SCROLL SOFT in your E11.INI

file if your display goes nuts a screen or so into each

attempted E11 session.

SET SWITCH n

SET SWITCH PORT n

If PORT is specified, specifies the octal 80x86 I/O address of a

word port which when read as a word, gives the current 16-bit

switch register value.  Otherwise (PORT not specified), sets the

value of the emulated SR to the octal number n.

SET LPn:  CSR=nnnnnn VECTOR=nnn

Sets the octal CSR and/or vector addresses for the specified

LP11 port (values will be saved for use when the port is created

using ASSIGN if it doesn't currently exist, otherwise effect is

immediate).

SET PR:  REWIND

Rewinds the file currently MOUNTed on PR:  (the PC11 reader) so

that subsequent input will start over at the beginning of the

file.

SET TTn:  CSR=nnnnnn VECTOR=nnn

Similar to SET LPn:, but for DL11 ports.

SET XEn:  BOOTSTRAP=DISABLE

SET XEn:  BOOTSTRAP=ddu:[/os]

Selects whether incoming MOP boot frames will be honored.

Default is DISABLE.  If a device (and optional OS switch) is

specified, incoming boot packets will act as a keyboard BOOT

command with those parameters.  Remote booting is not yet

supported, so this command has no visible effect yet.

SHOW CPU

Shows emulated CPU type, along with breakdown of features, as

well as the CPU type of the host processor (and whether the

Pentium FDIV bug is present).

SHOW DELAY device

Shows the currently active list of interrupt delay counts for

the specified device, starting with the delay for opcode number

0.  See SET DELAY for details.

SHOW MMU [ {KERNEL | SUPERVISOR | USER} [INSTRUCTION | DATA] ]

Shows the current mapping for the specified space.  Defaults are

kernel and instruction space.

SHOW ddu:

Shows the configuration of the specified device;  this is the

function key or COM/LPT port, CSR and vector if it's a DL11 or

LP11 port, or the disk type and write protect flag for disks, or

the station addresses and portal types for ethernet ports.

STEP

Executes a single instruction step and displays the updated

registers.

HISTORY:

V0.8 BETA, 29-Mar-1994;  initial release.

V0.9 BETA, 05-Jul-1994;  many bug fixes (mainly trap handling,

MMU emulation, DIV instruction, and VT100 reverse video), added

RX211 emulation, multiple DL11s, and 50 Hz KW11L mode.

V1.0 BETA, 14-Nov-1994;  more bug fixes, added FP11A,

RK611/RK06-07, LP11, D-space, and supervisor mode emulation.

Also CALC, HELP, INIT, LOG, SET/SHOW CPU, SET DR LPTn:, SET

SCROLL, SHOW MMU commands, VT100 graphics/underline, changed to

.EXE file (ran out of space in unified code/data segment in .COM

file).

V1.1 BETA, 22-Mar-1995;  still more bug fixes (IAS finally

works), DELUA ethernet emulation, disk LOGging, indirect command

files, workaround for Pentium FDIV bug, help text moved to file,

Russian HD:  device (and RT-11 driver), PC11, display general

registers on parallel port LED board.

NOTES:

This program is dedicated to RSTS V7.0-07 Digby's Bitpile.

Ad Majorem Digby Gloriam.

INTERRUPTS:

The interrupt system ended up having to be a little weird, due

to some shameful assumptions in DEC OSes (mainly RSX and RT-11

SJ) about how many instructions are guaranteed to be executed

after writing a command to a device CSR, before the device will

complete the operation and interrupt.  Since MS-DOS doesn't

support asynchronous I/O (unless you go to extremes which

wouldn't have made sense in a CPU hog like an instruction set

simulator), it's natural to have all emulated device I/O appear

to the PDP-11 to be instantaneous (not really, the PC takes a

break between PDP-11 instructions to do the transfer), with the

completion interrupt occurring on the instruction following the

one that started the transfer.  Unfortunately this catches some

OS code with its pants down;  in my testing, RSX appeared to

issue WAIT instructions for TTY output which was assumed not to

have completed yet a few dozen instructions after writing a

character to a DL11 (thus hanging the system), and similarly the

RT-11 SJ (but not FB/XM) keyboard interrupt service routine runs

with interrupts enabled on the assumption that another keyboard

interrupt couldn't possibly happen before the current ISR

finishes (when this does happen the ISR recurses and the

characters are put in the buffer in reverse order, which was

happening with VT100 keypad keys in E11).  My solution to these

problems was to put in a queueing system, so that the interrupt

(and in some cases the transfer itself) doesn't occur until a

set number of instruction fetches (which in most cases I set on

the order of hundreds) after the instruction that started the

transfer.  This slightly slows down I/O, and at some point I may

add commands to make each of the delays settable, for tuning in

cases where my hard-coded delays turn out not to be enough, or

cases of "clean" code which makes no assumptions in which case

things can be sped up by setting the all delays to 1.  I'd be

interested to see how these OSes handle really fast serial lines

on actual slow PDP-11s.  All of this applies to disks too,

although I haven't run into any cute problems yet of code that

executes within a buffer that a disk is reading to and counts on

the CPU's ability to "outrun" the disk and JMP out in time;  but

just to be safe the disk code delays actually doing the transfer

until it's about to post the interrupt anyway.  RK06/07 seek

attentions are delayed still further beyond acknowledging the

seek command so as not to confuse overlapped seek drivers;

however you'll get faster results using a non-overlapped driver

if one is supplied with your OS.  Since usually all your

emulated disks are on one physical DOS disk with one head

carriage, there's nothing to overlap anyway.

KEYBOARD:

The keypad layout may take a little getting used to but it's

intended to be familiar if your fingers are already comfortable

using KED or EDT on a real VT100;  just don't look at it.  The

digits and '.' key work as marked in Num Lock mode (otherwise

only the arrows work).  The keys around the top and right edges

of the keypad are *not* as marked, but correspond to the

PF1-PF4, hyphen, comma, and ENTER keys of the VT100 (the comma

key is missing unless you have an 84-key AT keyboard;  Northgate

Omnikey 102 keyboards have an = key where the VT100 comma

belongs but unfortunately it transmits the same scan code as the

=/+ key on the main keyboard so E11 can't use it as a comma).

To get the normal function of the Num Lock key (and Esc, Scroll

Lock, and Sys Req on an 84-key AT keyboard), press Alt, Ctrl, or

Shift at the same time.  The keypad hyphen, comma, ENTER, and

period keys are also available as F6, F8, F10, and F9 keys.

Backspace is CTRL/H, line feed is CTRL/J.

DISK IMAGES:

Getting a snapshot of a bootable disk for your OS into a huge

DOS file is Your Problem, I don't want to get involved in

pirating software from a company whose legal department is

probably bigger than any *company* I've ever worked for.  BTW,

my local (Albany, NY) DEC office has no conceptual problem with

selling a real license for a PDP-11 that doesn't really exist.

Anyway I used Kermit to slurp the disk images out of a real

PDP-11, and I've heard of people using Process Software TCP/IP

with a live PDP-11, or DECnet and Pathworks, or Mark Aitchison's

RT11.EXE (a fine program) with an RX50 distribution kit.

PUTR.COM, a companion program to E11, is available by anonymous

FTP from TATS.WIZVAX.COM, in the pub/putr directory and knows

how to read and write RT-11 format volumes on a variety of

media, as well as how to read RSTS/E and OS/8 formats.  It can

write blank container files with the serial numbers and (empty)

bad block files filled in correctly, which can be useful with

any OS.

Paul Koning (former RSTS developer) has written a very nice

program named "flx" for manipulating files in RSTS disk images,

among other things it can build a bootable disk given the files

from [0,1].  It's available from FTP.UPDATE.UU.SE in

pub/pdp11/rsts/utils/flx and is written in portable C, so it can

be used with any emulator (or with real disk packs on a VAX).  A

DOS executable is included with the sources.

*** WATCH OUT ***

The RT-11 DL:  and DM:  device handlers expect to find a bad

block replacement table in block 1 of a disk.  If something else

is there (like the pack label in Files-11 and RDS 1.1 and later,

or the MFD in RDS 0.0), they will replace blocks at random and

you'll get a corrupted disk image (yes, I'm sure :-).  So either

hack your Kermit (etc.)  to use the appropriate .SPFUN instead

of .READ, or don't use RT-11 to read non-RT-11 disks.

HOST SYSTEMS:

"Why didn't you write a version to run on the {machine} under

{OS}?"  The program is written entirely in 80x86 assembly

language (over 28,000 lines);  porting the devices, debugger,

interrupt system etc.  to another architecture would be

straightforward but the instruction set processor depends

heavily on similarities between the 80x86 and the PDP-11 (most

notably byte order and the condition flags;  correcting the byte

order and/or deriving condition flags "by hand" on another

machine would be very inefficient).  The FP11 simulation

likewise relies heavily on the 80x87 data formats.  Also the

VT100 emulator gets pretty intimate with the video hardware.  So

it seems a little unreasonable for you to ask me to spend months

MORE for no pay, writing the whole thing over again for your

machine, just to save you paying a couple of hundred bucks for

some cheesy mono 486DLC machine if you don't already have one.

Of course, if someone with a financial interest in keeping

PDP-11 code alive at above full speed wants to donate an Alpha

or PowerPC system to motivate me to port the whole mess to it,

that's another story!

Meanwhile, if what you want is a UNIX-based PDP-11 emulator in

C, three (or more?)  have already been released, by Bob Supnik

of DEC (J-11, in /pub/mbg/simulators on FTP.STD.COM), Eric

Edwards of RIT (11/40?  /pub/csh/mag/pdp.tar.Z on

FTP.CSH.RIT.EDU) and der Mouse of McGill (FP-less J-11?

/pub/people/mouse/pdp11 on FTP.CIM.MCGILL.CA).  You'll need a

faster machine to get the same results, but at least you get

portable source code.

SOURCES:

At this time I am not making a public release of the source

code, because it contains proprietary code from a possible

forthcoming commercial product.  Also the program is changing

rapidly and I like having all the bugs where I can see them.

COPYRIGHT NOTICE:

Ersatz-11 is Copyright (C) 1994, 1995 by John Wilson.  All

rights reserved.  Distribution of this document and/or the

E11.EXE executable file, in unmodified form, without charge, is

allowed without restriction.  Anything else is strictly

forbidden unless you contact me to work something out first.

ACKNOWLEDGMENTS:

That should cover most of the important stuff.  A lot of stuff

has gone right since the initial beta release, mostly thanks to

input either from people who have experience with PDP-11

emulation (Bob Supnik at DEC and Alan Sieving at QED), or from

people who have helped me debug Ersatz-11 with their

configuration (many people, Frank Borger's (U.  Chicago) efforts

with RT-11SJ and IAS have been particularly impressive, as have

Paul Koning's (Chipcom) insights into RSTS and Eduard Vopicka's

(Prague University of Economics) help with RSX).

FEEDBACK:

I would really appreciate continued feedback on the program,

good and bad;  bad news is usually more useful (I'm very eager

to fix bugs, but I have to find them first), but I'm always

happy to hear that things work right too, especially if you've

gotten it working with an OS that hasn't been tried before, or

if you've found a novel way to bootstrap a PDP-11 OS onto your

PC.  Also I'd be very interested in hearing suggestions for

improvements (besides the obvious 22-bit addressing).

AS ALWAYS, MAKE A BACKUP OF ANYTHING YOU'D REGRET LOSING BEFORE

RUNNING THIS PROGRAM.  Yeah I know you won't (I didn't) but

think about how you'll feel if something important gets

destroyed...  Anyway once you're set up it's a nice change being

able to throw a couple of dozen RL02 images on a QIC-80 tape for

safekeeping, might as well do your other data the same favor.

John Wilson

11 Bank Street

Troy, NY  12180

USA

+1 (518) 271-1982

wilsonj@rpi.edu           (most likely not to change soon)

wilson@tats.wizvax.com    (unless the hostname changes again)

John_Wilson@MTS.RPI.EDU   \

USERHA8G@RPITSMTS.BITNET  / (going away 06/30/95)